WAREHOUSE WIZARD WMS

Standard Work Card Pack

Lean one-page operating cards for 3PL warehouse teams

CLIENT TEMPLATE

Replace every {{PLACEHOLDER}} with your site's values before issuing to the floor.

| Field | Value |
| --- | --- |
| Facility / Client | {{CLIENT_NAME}} — {{WAREHOUSE_CODE}} |
| Document owner | {{DOC_OWNER}} |
| Version / Rev | {{VERSION}} • Rev {{REV}} |
| Effective date | {{EFFECTIVE_DATE}} |
| Next review | {{REVIEW_DATE}} |
| Approved by | {{APPROVER}} |

Contents

# How to use this pack

One card = one task = one page. Print single-sided, laminate, and post each card at its point of use (dock, put-away aisle, pick zone, dispatch bay, count station). Operators should be able to run the task from the card without leaving the floor.

## Placeholder key

Every value in double braces is site-specific. Do a find-and-replace across this document before you issue it:

| Placeholder | Replace with |
| --- | --- |
| {{CLIENT_NAME}} / {{WAREHOUSE_CODE}} | Client and facility code shown on labels and pick lists |
| {{SCANNER_MODEL}} / {{MHE}} | Handheld/tablet model and material-handling equipment used |
| {{PPE}} | Required personal protective equipment for the task/zone |
| {{SUPERVISOR}} / {{ESCALATION}} | Who to call and the escalation channel when work stops |
| {{VARIANCE_THRESHOLD}} | Cycle-count variance that must be escalated (e.g. any, >2%) |
| {{COUNT_CADENCE}} | How often A/B/C or high-value stock is counted |
| {{SHIFT_TIMES}} / {{LABEL_PRINTER}} | Shift pattern and label printer for reviews and setup |

## Conventions on every card

- Scan-confirmed: the WMS is only satisfied by a real scan. If it blocks you, that is the process working — do not type a value just to move on.

- STOP / ANDON (amber box): the conditions that must halt the task. Keep the stock where it is and escalate.

- Standard work (blue footer): the one line that must be true every shift, plus the in-app Help article for detail.

## Revision control

Treat this as a controlled document. Update the Rev and dates on the cover whenever a step changes, re-issue the affected card(s), and remove superseded printouts from the floor so no one runs an old process.

# Lean standard work — the why behind the cards

The WMS is built so the correct action is the easiest action. Scan prompts, role-specific screens, sign-offs, and audit logs exist to remove ambiguity, not to add bureaucracy. Standard work means every shift receives, puts away, picks, transfers, counts, and changes status the same controlled way — unless a supervisor deliberately changes the process.

## The eight wastes these cards attack

Searching, walking, waiting, re-keying, re-checking, correcting, over-processing, and explaining after the fact. Role-specific screens cut over-processing; badge/PIN login cuts time at shared devices; driver sign-off makes transport errors visible; scan-confirmation cuts defects and rework.

## 5S on the floor

5S becomes real when locations are named, labels scan, blocked stock is visible, and each person owns sustaining the process. Use the dashboard exceptions — blocked locations, quarantine buildup, missing stock, repeated scan failures — as a daily 5S audit signal for where standard work is slipping.

## Andon — stop the line

The amber STOP box on each card is an Andon. When the floor disagrees with the system, stopping the transaction is cheaper than pushing a bad one through. Verify in Inventory Search, apply a controlled status when authorized, and log the exception.

## How the WMS maps to lean

| WMS feature | Lean purpose |
| --- | --- |
| Scan-confirmed workflows | Poka-yoke / error-proofing — the wrong action is hard to complete |
| Whole-pallet picking | Standard work — no ad-hoc splitting or partial-count guesswork |
| Directed put-away & bay grid | Cuts searching and walking; enforces temperature compatibility |
| Driver departure sign-off | Andon quality gate before stock goes in-transit |
| Cycle counts & variances | DMAIC / DPMO signal — variance = a broken upstream handoff |
| Status controls with reasons | Makes restricted stock visible and auditable |
| Non-destructive user removal | Preserves audit history; least-privilege access |

## Acronyms

FEFO First Expired First Out • FIFO First In First Out • 5S Sort, Set-in-order, Shine, Standardize, Sustain • Andon visual stop signal • DMAIC Define, Measure, Analyze, Improve, Control • DPMO Defects Per Million Opportunities • MHE Material-Handling Equipment • PPE Personal Protective Equipment.

# Card index by role

Fifteen cards, grouped by the role that owns the task. Give each operator the cards for their role; give supervisors the full pack.

| Card | Task | Role |
| --- | --- | --- |
| SW-01 | Directed Put-Away | WAREHOUSE OPERATOR |
| SW-02 | Whole-Pallet Picking | WAREHOUSE OPERATOR |
| SW-03 | Location Move | WAREHOUSE OPERATOR |
| SW-04 | Cycle Count | WAREHOUSE OPERATOR |
| SW-05 | Receiving – Inbound Shipment | INVENTORY CLERK |
| SW-06 | Product & Packaging Master Data | INVENTORY CLERK |
| SW-07 | Status Control – Hold / Quarantine / Damaged / Missing | INVENTORY CLERK |
| SW-08 | Inventory Search & Verify | INVENTORY CLERK |
| SW-09 | Transfer Dispatch – Departure Sign-Off | DISPATCH DRIVER |
| SW-10 | Transfer Receive & Handoff | DISPATCH DRIVER |
| SW-11 | Pick List Release & Work Planning | MANAGER / ADMIN |
| SW-12 | Exception & Andon Response | MANAGER / ADMIN |
| SW-13 | Users, Roles & Badges | MANAGER / ADMIN |
| SW-14 | Warehouse Setup & Structure | MANAGER / ADMIN |
| SW-15 | Reports & Daily Operational Review | MANAGER / ADMIN |

THE CARDS

Each card below prints on its own page.

| SW-01 Directed Put-Away WAREHOUSE OPERATOR |  |
| --- | --- |
| Module | Put-Away |
| Trigger | Pallets released to put-away from Receiving or a Transfer receipt; your queue shows a put-away task. |
| Before you start | Signed in by badge/PIN on {{SCANNER_MODEL}}. Correct facility {{WAREHOUSE_CODE}} selected. Wearing {{PPE}}; pallet physically staged. |
| Steps | 1 Open your Put-Away queue and select the next task. 2 Scan the pallet barcode to confirm identity. 3 Scan the location: a full code (e.g. A-05-L05) fills the field directly; a short bay code opens the bay grid. 4 If the bay grid opens, tap the exact available cell shown — a bay code is not a final location. 5 Confirm. Stock moves to stored & available and the move is audit-logged. |
| STOP / ANDON | Location is inactive, full, or temperature-incompatible (e.g. cool stock → ambient cell). Pallet or location will not scan, or the floor disagrees with the system. Do NOT force a value. Leave the pallet where it is and escalate to {{SUPERVISOR}}. |
| Done when | Both pallet and location scans are confirmed and the task clears from your queue. |
| Common errors | Scanning the wrong location. Treating a bay code as a final location. Storing cool stock in an ambient location. |
| Standard work: Two scans, every pallet, every time — pallet, then location. In-app Help: Put-Away → Directed Put-Away |  |

| SW-02 Whole-Pallet Picking WAREHOUSE OPERATOR |  |
| --- | --- |
| Module | Pick Lists / Pick Execution |
| Trigger | A pick task is assigned to you from a released pick list. |
| Before you start | Signed in; correct {{WAREHOUSE_CODE}} selected. Wearing {{PPE}}; {{MHE}} ready. Picks are whole-pallet only — no splitting. |
| Steps | 1 Open Pick Execution and read the directed location (rack, aisle, bay, level, position). 2 Travel there. Scan the bay code to open the bay grid (assigned cell highlighted), or scan the exact location code to fill it directly. 3 Scan the pallet barcode — the Confirm pick button flashes yellow. 4 Confirm the pick before doing anything else on that task. 5 Move the whole pallet to the directed staging / dispatch lane. |
| STOP / ANDON | Whole-pallet only: if the system expects 100 and only 80/90 are present, do NOT confirm a short pick. Pallet missing, damaged, or inaccessible — stop; notify {{SUPERVISOR}}. Use Inventory Search + Status Controls to restrict the pallet; add a System Log note. |
| Done when | Pallet scanned, pick confirmed, and pallet staged at the directed lane. |
| Common errors | Forcing/keying a partial quantity to move on. Splitting a pallet to match demand. Confirming the wrong cell after a bay scan. |
| Standard work: One pallet = one task. Never key a partial to move forward — raise it as an exception. In-app Help: Pick Lists → Operational Dead-Ends |  |

| SW-03 Location Move WAREHOUSE OPERATOR |  |
| --- | --- |
| Module | Location Moves |
| Trigger | A stored pallet must move to a new bin, staging, or picking slot inside the SAME warehouse. |
| Before you start | Signed in; correct {{WAREHOUSE_CODE}} selected. Wearing {{PPE}}. Confirm the pallet is staying in this warehouse (else use Transfers). |
| Steps | 1 Open Location Moves. Scan or enter the pallet barcode. 2 Scan, type, or Browse bays to choose the destination location. 3 Confirm the destination and complete the scan-confirmed move. 4 Cancel a queued or in-progress move from the task list if the plan changes. |
| STOP / ANDON | Destination is disabled, full, or temperature-incompatible. Pallet is actually changing warehouse — use Transfers, not Moves. Stop and correct the destination before posting. |
| Done when | Pallet and destination confirmed; the move posts with pallet identity and audit history intact. |
| Common errors | Using Transfers for a same-warehouse move. Confirming a bay code as the final location instead of picking an exact bin. |
| Standard work: Same warehouse = Location Move; different warehouse = Transfer. In-app Help: Location Moves |  |

| SW-04 Cycle Count WAREHOUSE OPERATOR |  |
| --- | --- |
| Module | Cycle Counts |
| Trigger | A count sheet is generated for a location, zone, SKU, or spot check (cadence: {{COUNT_CADENCE}}). |
| Before you start | Signed in; correct {{WAREHOUSE_CODE}} selected. Wearing {{PPE}}. Count the physical stock — never from memory. |
| Steps | 1 Open your assigned count sheet. 2 Go to the location and scan / verify the location. 3 Physically count the stock present. 4 Enter the counted quantity — the system updates stock and creates an adjustment record if there is a variance. 5 Submit; flag exceptions for investigation. |
| STOP / ANDON | Variance beyond {{VARIANCE_THRESHOLD}}, a missing pallet, or a wrong status. System and floor disagree — do not overwrite blindly; report to {{SUPERVISOR}} for root cause. |
| Done when | Counted quantity submitted and any variance recorded for follow-up. |
| Common errors | Counting from memory or the expected figure. Ignoring variance thresholds. |
| Standard work: Count what you see, not what you expect. A variance is a broken handoff upstream — DMAIC it. In-app Help: Cycle Counts |  |

| SW-05 Receiving – Inbound Shipment INVENTORY CLERK |  |
| --- | --- |
| Module | Receiving |
| Trigger | A container / PO arrives to be booked in under a client owner. |
| Before you start | Signed in; correct {{WAREHOUSE_CODE}} selected. Client ownership confirmed; camera/scanner ready. Cool-chain items have a cool zone + packaging profile set. |
| Steps | 1 New Shipment: confirm warehouse context, then container number. Use Scan container number to OCR the ISO 6346 code and confirm only the GREEN candidate — focus then jumps to PO. 2 Enter the PO number. 3 Scan product or search by SKU/name to select, then press the right-arrow commit button to lock the product for that line. 4 Enter Total received (Enter) → Qty per pallet (Enter) → Pallets (Enter) → expiry calendar. Treat any learned qty suggestion as a time-saver, not the count. 5 Add lot / batch / packaging as needed, verify all fields, then Save / Receive. Print draft pallet labels and send pallets to Put-Away. |
| STOP / ANDON | Container will not turn green; SKU unknown; cool-chain item with no cool zone/profile; counts disagree with paperwork. Stop before saving — fix master data / setup with {{SUPERVISOR}}. |
| Done when | Pallets created with lot/expiry, labels printed, and the put-away workload generated. |
| Common errors | Inserting a container number before the green ISO 6346 candidate. Skipping the product commit arrow. Letting a learned suggestion replace the physical count. |
| Standard work: Scan → commit → count → expiry, in that order. The physical count always wins. In-app Help: Receiving → Label Printing |  |

| SW-06 Product & Packaging Master Data INVENTORY CLERK |  |
| --- | --- |
| Module | Products / Packaging Profiles |
| Trigger | A new SKU, client, or pack configuration must exist before stock can be received. |
| Before you start | Signed in as clerk or manager. Correct client owner identified. Barcodes and pack data supplied by the client. |
| Steps | 1 Create the Product: SKU identity, client ownership, rotation method (FEFO/FIFO), temperature and handling flags. 2 Set tracking flags (expiry, lot) to match how the product is controlled. 3 Create a Packaging Profile: each / case / pallet sizes, dimensions, barcodes — one default per product. 4 Align barcodes to the physical packaging and apply labels. 5 Hide (do not delete) discontinued products/profiles to preserve audit history. |
| STOP / ANDON | Missing or duplicate barcode, unclear ownership, or temperature settings that conflict with available zones. Resolve the master data before any warehouse work is released. |
| Done when | Product plus a single default packaging profile are active and receivable. |
| Common errors | Wrong rotation method. Temperature requirement inconsistent with storage zones. More than one default profile for a product. |
| Standard work: Get master data right once — every downstream scan depends on it. In-app Help: Products / Packaging Profiles |  |

| SW-07 Status Control – Hold / Quarantine / Damaged / Missing INVENTORY CLERK |  |
| --- | --- |
| Module | Statuses |
| Trigger | Stock must be restricted, isolated, or flagged while staying auditable. |
| Before you start | Signed in as clerk or manager. Pallet identified in Inventory Search. A real, recordable reason for the change. |
| Steps | 1 Open the pallet in Inventory Search, then open Status Controls. 2 Choose the status: Hold (needs review, disposition unknown); Quarantine (quality / expiry / contamination / temperature / customer isolation); Damaged or Missing only after the condition/count is verified. 3 Record a clear reason — required; it drives downstream decisions. 4 Return stock to Available only when authorized and verified. 5 Raise a System Log entry for anything needing quality, customer, or manager follow-up. |
| STOP / ANDON | If you cannot verify condition or count, use Hold — do not mark Damaged/Missing on assumption. |
| Done when | Status applied with a reason and the pallet is visible as controlled stock. |
| Common errors | Changing status without reason detail. Forgetting that controlled stock still affects downstream decisions. |
| Standard work: Every status change carries a reason and an audit trail. In-app Help: Status Controls → Operational Dead-Ends |  |

| SW-08 Inventory Search & Verify INVENTORY CLERK |  |
| --- | --- |
| Module | Inventory Search |
| Trigger | You need to confirm live stock state before a pick, move, transfer, or exception decision. |
| Before you start | Signed in. Know the SKU / pallet / lot / location you are checking. |
| Steps | 1 Search by SKU, pallet, lot, warehouse, or location. 2 Open the pallet detail: quantity, status, lot context, and recent movement history. 3 Validate the record against the physical floor before acting. 4 Act on the live record — not memory or a stale note. |
| STOP / ANDON | Record and floor disagree — do not proceed with the pick/move; escalate for correction via Location Moves, Status, or a Cycle Count. |
| Done when | Live stock state confirmed and the next action is justified by the record. |
| Common errors | Ignoring status filters. Acting on stale assumptions instead of the live record. |
| Standard work: Verify in the system before you touch the pallet. In-app Help: Inventory Search |  |

| SW-09 Transfer Dispatch – Departure Sign-Off DISPATCH DRIVER |  |
| --- | --- |
| Module | Transfers |
| Trigger | A transfer is created and a pallet is ready to physically leave for another facility. |
| Before you start | Signed in with your badge / user code. Route and destination verified. Correct pallet(s) identified for this load. |
| Steps | 1 Open the transfer; confirm source warehouse, destination, and pallet identity. 2 Physically load the correct pallet onto the correct vehicle. 3 Scan your badge / enter your user code to sign the handoff. 4 Sign-off sets the pallet to in-transit — a signed record that stock has departed. |
| STOP / ANDON | Wrong pallet, wrong vehicle, missing paperwork, or route mismatch. Stop visibly — do NOT dispatch a bad load. Escalate to {{SUPERVISOR}} while it is still cheap to fix. |
| Done when | Driver has signed off and the pallet shows in-transit. |
| Common errors | Dispatching the wrong pallet. Letting the system show in-transit while the pallet is still in staging or on the wrong vehicle. |
| Standard work: Scan pallet → validate route → driver sign-off → in-transit. Seconds now prevent lost loads later. In-app Help: Transfers |  |

| SW-10 Transfer Receive & Handoff DISPATCH DRIVER |  |
| --- | --- |
| Module | Transfers |
| Trigger | An in-transit transfer arrives at the destination facility. |
| Before you start | Signed in; correct destination {{WAREHOUSE_CODE}} selected. Destination structure is ready to store the stock. |
| Steps | 1 Open the inbound transfer at the destination. 2 Confirm pallet identity and lot against the transfer record. 3 Receive the pallet into the destination. 4 Immediately trigger directed Put-Away so stock does not sit invisible in a receiving lane. |
| STOP / ANDON | Pallet or lot does not match the transfer, or the destination structure is not ready — stop and reconcile before receiving. |
| Done when | Pallet received and handed to Put-Away with identity and audit history intact. |
| Common errors | Receiving before the destination structure is ready. Leaving received stock stranded in the receiving lane. |
| Standard work: Receive, then put away — never strand stock in the lane. In-app Help: Transfers → Put-Away |  |

| SW-11 Pick List Release & Work Planning MANAGER / ADMIN |  |
| --- | --- |
| Module | Pick Lists |
| Trigger | Outbound demand needs to be released as executable pick work. |
| Before you start | Manager role. Stock confirmed available in Inventory / Reports. Staging and dispatch zones ready. |
| Steps | 1 Create the pick list — the system generates whole-pallet tasks from available palletized inventory. 2 Review shortages before releasing; release only when the stock exists. 3 Release the work and assign / monitor via the execution detail. 4 Handle short, damaged, or missing pallets as exceptions (hold / status / log) — never as forced confirmations. |
| STOP / ANDON | Demand exceeds available pallets, or a task would require splitting a pallet — resolve the stock state first. |
| Done when | Pick tasks released against real available stock and being executed. |
| Common errors | Releasing before stock is available. Treating order demand as permission to split a pallet. |
| Standard work: Do not release work the floor cannot finish. In-app Help: Pick Lists / Reports |  |

| SW-12 Exception & Andon Response MANAGER / ADMIN |  |
| --- | --- |
| Module | Help → Dead-Ends |
| Trigger | An operator stops a task: won't-scan, wrong location, quantity mismatch, damage, blocked cell, unknown barcode, or hardware/network failure. |
| Before you start | Manager/clerk access to Inventory Search, Status Controls, and System Log. |
| Steps | 1 Keep the stock where it is; verify pallet, location label, warehouse selector, and task number. 2 Diagnose in Inventory Search (last known location, movement history). 3 Apply the right control: Location Move (misplaced), Status hold/quarantine/damaged/missing (condition), or reprint the label (damaged/stale code). 4 For a short whole-pallet pick: stop the pick, place the pallet on a controlled status, and record a System Log note (pending the Pick Exception Resolver). 5 Log anything needing quality/customer/manager follow-up; resolve the log only after the real issue is fixed. |
| STOP / ANDON | Unidentified pallet or a system/floor conflict — never receive, move, pick, or ship under a guessed code. Investigate first. |
| Done when | Stock state corrected in the system, audit + log written, and the operator is unblocked. |
| Common errors | Working around a scan to move on. Resolving a System Log entry before the underlying issue is fixed. |
| Standard work: Stop-the-line beats a bad transaction. Verify, control, log. In-app Help: Operational Dead-Ends |  |

| SW-13 Users, Roles & Badges MANAGER / ADMIN |  |
| --- | --- |
| Module | Users & Roles |
| Trigger | Onboarding, a role change, or access removal for a warehouse user. |
| Before you start | Admin access. Public self-signup is disabled — admins create accounts. |
| Steps | 1 Add the user (Admin/Dev creates the account and approves the profile). 2 Assign the smallest role that fits the work: warehouse_operator, inventory_clerk, dispatch_driver, warehouse_manager, or admin. 3 Issue a short user code and, for tablets, a scannable badge code + PIN (a password is still required). 4 To remove one area of access, HIDE the role assignment; to stop sign-in entirely, DISABLE the profile. 5 Never delete — hide/restore so audit history is preserved. |
| STOP / ANDON | If a change would strip access from someone mid-task, reassign the work first. |
| Done when | User has the correct least-privilege role and working credentials; every change is in the activity log. |
| Common errors | Removing the wrong role assignment. Disabling a profile before reassigning operational work. |
| Standard work: Least privilege, non-destructive removal, every change audited. In-app Help: Users, Roles, Badges & Access |  |

| SW-14 Warehouse Setup & Structure MANAGER / ADMIN |  |
| --- | --- |
| Module | Setup Wizard / Settings |
| Trigger | Standing up a new facility, or extending / rebuilding structure after a Reset All. |
| Before you start | Admin/manager access. Physical racking layout known. Label printer ready ({{LABEL_PRINTER}}). |
| Steps | 1 Step 1 — define facilities: short uppercase code, name, city, country; mark cool-zone capability. 2 Step 2 — define zones (Receiving, Put-Away, Picking, Cool, Quarantine, Bulk) with temperature class; flag no-stock zones. 3 Step 3 — generate bin locations: aisles, bays, levels, depth 1–5; confirm the code pattern; set capacity / stack limits. 4 Step 4 — review totals; go back to fix anything before it is written (nothing is created until Step 5). 5 Step 5 — create the structure in a single transaction, then print beam / bay / zone labels so physical racks match the scan codes. |
| STOP / ANDON | Location codes that do not match the physical racks, or editing structure without reprinting labels — fix before operators are sent there. |
| Done when | Warehouses, zones, and bins exist and the physical labels match the system codes. |
| Common errors | Inconsistent warehouse codes across steps. Location rules that do not match the zone's purpose. Running Reset All without a rebuild plan. |
| Standard work: Structure first, labels to match, then release work. In-app Help: Warehouse Setup / Structure Tool |  |

| SW-15 Reports & Daily Operational Review MANAGER / ADMIN |  |
| --- | --- |
| Module | Reports / Dashboard / System Log |
| Trigger | Shift start / end review, or monitoring flow through the day ({{SHIFT_TIMES}}). |
| Before you start | Manager access to Dashboard, Reports, and System Log. |
| Steps | 1 Open the Dashboard: inbound, storage, and outbound activity; open work; occupancy trend. 2 Read exceptions as a daily 5S signal: blocked locations, quarantine buildup, missing stock, repeated scan failures. 3 Check Reports: stock by warehouse, occupancy pressure, recent movements / audit. 4 Review System Log; assign owners; resolve only closed issues; snapshot record counts after setup / reset / import. 5 Act on bottlenecks before releasing more work into them. |
| STOP / ANDON | Rising quarantine / missing / blocked trends — treat as standard work slipping and investigate root cause. |
| Done when | Exceptions triaged, owners assigned, and the day's plan reflects real conditions. |
| Common errors | Treating snapshots as a substitute for transactional detail. Ignoring audit trends. |
| Standard work: The dashboard is your daily 5S audit — manage the exceptions, not the averages. In-app Help: Reports / Enterprise Dashboard Modes |  |